## Exporting CAD data from Cadence Allegro

## Cadence Allegro running on Unix

- In order to extract ASCII cad files from Cadence Allegro running on UNIX, CircuitCAM provides two versions of script and command files.  These files must be used with Allegro’s EXPORT utility.

- XTRCT.SCR (XTRCT.CMD): These files make up the standard script which produces ASCII cad files without the net list information.  This is the preferred script because the resulting ASCII files will be much smaller than those produced with the XTRCTNET.SCR script.

- XTRCTNET.SCR (XTRCTNET.CMD): These files make up the script which produces ASCII cad files with the net list information.  This script will produce a much larger ASCII file than those from the XTRCT.SCR script.  Consequently, the corresponding CircuitCAM project file will also be much larger.  If net list information is required, then this script must be used.

- Before running a script, the script and command files must be converted to UNIX format.  This is done with the DOS2UNIX program.  After conversion, place the script and command files in the /bin or /local/bin directory.  Then change the script file permission to executable (mode = 777).

- To activate the script file, type XTRCT.SCR (or XTRCTNET.SCR) followed by the board design file name.  The script file will use the corresponding command file and produce an ASCII cad file.  The generated file will be CCAM.CAD (XTRCT.SCR) or CCAMNET.CAD (XTRCTNET.SCR).

- The generated file should then be renamed to represent the board design file from which it was created.

## Cadence Allegro running on Windows NT

- In order to extract ASCII cad files from Cadence Allegro running on Windows NT, CircuitCAM provides two versions of batch and command files.  These files must be used with Allegro’s EXPORT utility.

- XTRCT.BAT (XTRCT.CMD): These files make up the standard script which produces ASCII cad files without the net list information.  This is the preferred script because the resulting ASCII files will be much smaller than those produced with the XTRCTNET.BAT script.

- XTRCTNET.BAT (XTRCTNET.CMD): These files make up the script which produces ASCII cad files with the net list information.  This script will produce a much larger ASCII file than those from the XTRCT.BAT script.  Consequently, the corresponding CircuitCAM project file will also be much larger.  If net list information is required, then this script must be used.

- Before running a script, place the batch and command files in the current working directory of the Allegro database.

- To activate the script file, type XTRCT.BAT (or XTRCTNET.BAT) followed by the board design file name at the DOS prompt.  The script file will use the corresponding command file and produce an ASCII cad file.  The generated file will be CCAM.CAD (XTRCT.BAT) or CCAMNET.CAD (XTRCTNET.BAT).

- The generated file should then be renamed to represent the board design file from which it was created.