(lvsRules

procedure((parallelMos m1 m2)

prog((mt)

(mt = ncons(nil))

if(((m1->l) && (m2->l))

(mt->l = ((float((m1->l)) + float((m2->l))) / 2.0))

)

if(((m1->m) && (m1->w))

(m1->w = (float((m1->w)) * float((m1->m))))

)

if(((m2->m) && (m2->w))

(m2->w = (float((m2->w)) * float((m2->m))))

)

if(((m1->w) && (m2->w))

(mt->w = (float((m1->w)) + float((m2->w))))

)

mt->m=1

return(mt)

)

)

procedure((parallelRFMos m1 m2)

prog((mt)

(mt = ncons(nil))

if(((m1->lr) && (m2->lr))

(mt->lr = ((float((m1->lr)) + float((m2->lr))) / 2.0))

)

if(((m1->nr) && (m1->wr))

(m1->wr = (float((m1->wr)) * float((m1->nr))))

)

if(((m2->nr) && (m2->wr))

(m2->wr = (float((m2->wr)) * float((m2->nr))))

)

if(((m1->wr) && (m2->wr))

(mt->wr = (float((m1->wr)) + float((m2->wr))))

)

mt->nr=1

return(mt)

)

)

procedure((parallelNMOSCap m1 m2)

prog((mt)

(mt = ncons(nil))

if(((m1->lr) && (m2->lr))

(mt->lr = ((float((m1->lr)) + float((m2->lr))) / 2.0))

)

if(((m1->mr) && (m1->wr))

(m1->wr = (float((m1->wr)) * float((m1->mr))))

)

if(((m2->mr) && (m2->wr))

(m2->wr = (float((m2->wr)) * float((m2->mr))))

)

if(((m1->wr) && (m2->wr))

(mt->wr = (float((m1->wr)) + float((m2->wr))))

)

mt->mr=1

return(mt)

)

)

procedure((parallelCap m1 m2)

prog((mt)

(mt = ncons(nil))

if(((m1->c) && (m2->c))

(mt->c = (float((m1->c)) + float((m2->c))))

)

if(((m1->a) && (m2->a))

(mt->a = (float((m1->a)) + float((m2->a))))

)

return(mt)

)

)

procedure((seriesCap m1 m2)

prog((mt)

(mt = ncons(nil))

if(((m1->c) && (m2->c))

(mt->c = ((float((m1->c)) * float((m2->c))) / (float((m1->c)) + float((m2->c)))))

)

if(((m1->a) && (m2->a))

(mt->a = ((float((m1->a)) * float((m2->a))) / (float((m1->a)) + float((m2->a)))))

)

return(mt)

)

)

procedure((parallelRes m1 m2)

prog((mt)

(mt = ncons(nil))

if(((m1->r) && (m2->r))

(mt->r = ((float((m1->r)) * float((m2->r))) / (float((m1->r)) + float((m2->r)))))

)

return(mt)

)

)

procedure((seriesRes m1 m2)

prog((mt)

(mt = ncons(nil))

if(((m1->r) && (m2->r))

(mt->r = (float((m1->r)) + float((m2->r))))

)

return(mt)

)

)

procedure((compareMosLW m1 m2)

prog((x length width)

(x = nil)

(length = nil)

(width = nil)

if(((m1->m) && (m1->w))

(m1->w = (float((m1->w)) * float((m1->m))))

)

if(((m2->m) && (m2->w))

(m2->w = (float((m2->w)) * float((m2->m))))

)

if(((m1->w) && (m2->w)) then

if((abs(((m1->w) - (m2->w))) > (0.02 * (m2->w))) then

sprintf(width "MOS width mismatch: %g(extracted) vs %g(schematic)."

float((m1->w))

float((m2->w))

)

) else

sprintf(width "no MOS width specified in schematic or extracted.")

)

if(((m1->l) && (m2->l)) then

if((abs(((m1->l) - (m2->l))) > (0.02 * (m2->l))) then

sprintf(length "MOS length mismatch: %g(extracted) vs %g(schematic).\n"

float((m1->l))

float((m2->l))

)

) else

sprintf(length "no MOS length specified in schematic or extracted. \n")

)

if(width then

(x = width)

when(length

(x = strcat(width length))

) else

when(length

(x = length)

)

)

return(x)

)

)

procedure((compareRFMosLW m1 m2)

prog((x length width)

(x = nil)

(length = nil)

(width = nil)

if(((m1->nr) && (m1->wr))

(m1->wr = (float((m1->wr)) * float((m1->nr))))

)

if(((m2->nr) && (m2->wr))

(m2->wr = (float((m2->wr)) * float((m2->nr))))

)

if(((m1->wr) && (m2->wr)) then

if((abs(((m1->wr) - (m2->wr))) > (0.02 * (m2->wr))) then

sprintf(width "MOS width mismatch: %g(extracted) vs %g(schematic)."

float((m1->wr))

float((m2->wr))

)

) else

sprintf(width "no MOS width specified in schematic or extracted.")

)

if(((m1->lr) && (m2->lr)) then

if((abs(((m1->lr) - (m2->lr))) > (0.02 * (m2->lr))) then

sprintf(length "MOS length mismatch: %g(extracted) vs %g(schematic).\n"

float((m1->lr))

float((m2->lr))

)

) else

sprintf(length "no MOS length specified in schematic or extracted. \n")

)

if(width then

(x = width)

when(length

(x = strcat(width length))

) else

when(length

(x = length)

)

)

return(x)

)

)

procedure((compareNMOSCapLW m1 m2)

prog((x length width)

(x = nil)

(length = nil)

(width = nil)

if(((m1->mr) && (m1->wr))

(m1->wr = (float((m1->wr)) * float((m1->mr))))

)

if(((m2->mr) && (m2->wr))

(m2->wr = (float((m2->wr)) * float((m2->mr))))

)

if(((m1->wr) && (m2->wr)) then

if((abs(((m1->wr) - (m2->wr))) > (0.02 * (m2->wr))) then

sprintf(width "MOS width mismatch: %g(extracted) vs %g(schematic)."

float((m1->wr))

float((m2->wr))

)

) else

sprintf(width "no MOS width specified in schematic or extracted.")

)

if(((m1->lr) && (m2->lr)) then

if((abs(((m1->lr) - (m2->lr))) > (0.02 * (m2->lr))) then

sprintf(length "MOS length mismatch: %g(extracted) vs %g(schematic).\n"

float((m1->lr))

float((m2->lr))

)

) else

sprintf(length "no MOS length specified in schematic or extracted. \n")

)

if(width then

(x = width)

when(length

(x = strcat(width length))

) else

when(length

(x = length)

)

)

return(x)

)

)

procedure((compareCapValue m1 m2)

prog((x)

(x = nil)

if(((m1->c) && (m2->c))

if((abs(((m1->c) - (m2->c))) > (0.02 * (m2->c)))

sprintf(x "capacitor value mismatch : %g(extracted) vs %g(schematic). \n"

float((m1->c))

float((m2->c))

)

)

sprintf(x "no cap value specified in schematic or extracted. \n")

)

return(x)

)

)

procedure((compareResValue m1 m2)

prog((x)

(x = nil)

if(((m1->r) && (m2->r))

if((abs(((m1->r) - (m2->r))) > (0.02 * (m2->r)))

sprintf(x "resistor value mismatch: %g(extracted) vs %g(schematic). \n"

float((m1->r))

float((m2->r))

)

)

sprintf(x "no resistor value specified in schematic or extracted. \n")

)

return(x)

)

)

(permuteDevice parallel "nfet" parallelMos)

(permuteDevice parallel "nfet_rf" parallelRFMos)

(permuteDevice parallel "pfet_rf" parallelRFMos)

(permuteDevice parallel "3Vpfet_rf" parallelRFMos)

(permuteDevice parallel "3Vnfet_rf" parallelRFMos)

(permuteDevice parallel "nfet_na" parallelMos)

(permuteDevice parallel "pfet" parallelMos)

(permuteDevice parallel "3Vnfet" parallelMos)

(permuteDevice parallel "3Vnfet_na" parallelMos)

(permuteDevice parallel "3Vpfet" parallelMos)

(permuteDevice parallel "capacitor" parallelCap)

(permuteDevice series "capacitor" seriesCap)

(permuteDevice parallel "mimcap" parallelCap)

(permuteDevice parallel "nmoscap" parallelNMOSCap)

(permuteDevice parallel "3Vnmoscap" parallelNMOSCap)

(permuteDevice series "mimcap" seriesCap)

(permuteDevice parallel "resistor" parallelRes)

(permuteDevice series "resistor" seriesRes)

(pruneDevice general ("pfet_rf" "nfet_rf" "3Vpfet_rf" "3Vnfet_rf" "pfet" "nfet" "nfet_na" "3Vnfet" "3Vpfet" "3Vnfet_na"))

(compareDeviceProperty "pfet" compareMosLW)

(compareDeviceProperty "pfet_rf" compareRFMosLW)

(compareDeviceProperty "nfet_rf" compareRFMosLW)

(compareDeviceProperty "3Vnfet_rf" compareRFMosLW)

(compareDeviceProperty "3Vpfet_rf" compareRFMosLW)

(compareDeviceProperty "nmoscap" compareNMOSCapLW)

(compareDeviceProperty "3Vnmoscap" compareNMOSCapLW)

(compareDeviceProperty "nfet" compareMosLW)

(compareDeviceProperty "nfet_na" compareMosLW)

(compareDeviceProperty "3Vnfet" compareMosLW)

(compareDeviceProperty "3Vnfet_na" compareMosLW)

(compareDeviceProperty "3Vpfet" compareMosLW)

(compareDeviceProperty "capacitor" compareCapValue)

(compareDeviceProperty "mimcap" compareCapValue)

(compareDeviceProperty "resistor" compareResValue)

)